PROGRAM PARAMETERS THAT ARE CONTROLED FROM THE PANEL DISPLAY

(IN “SMS-DL06” program ONLY )             file: DL 06 Parameters

| Parameter | Value | component | Symbol | Code |
| --- | --- | --- | --- | --- |
| Main cycle (12 min) | 7200 | T6 | V4000 | V4020 |
| Shredder “on” and “off” period of time (90 sec) | 900 | T21 | V4001 | V4021 |
| Shredder on timer (4 sec) | 40 | T22 | V4002 | V4022 |
| Shredder off timer (8 sec) | 80 | T23 | V4003 | V4023 |
| Number of press actions (5) | 6 | CNT3 | V4004 | V4024 |
| Number of machine wash (1) | 2 | CNT1 | V4005 | V4025 |
| Water addition timer before valve opens (8 sec) | 80 | T14 | V4006 | V4026 |
| Closing discharge valve timer (20) | 200 | T10 | V4007 | V4027 |
| Metering pump timer (30/26 sec) | 300/260 | T3 | V4010 | V4030 |
| Separator operation timer (90 sec) | 900 | T13 | V4011 | V4031 |
| High level safety timer (according to filling time) | 600 | T26 | V4012 | V4032 |
| Cycles record (to erase use “close” button) | 10 sec |  | V4013 | V4033 |
| Separator wash solenoid timer (60 sec) | 600 | T100 | V4014 | V4034 |
| Water fill timer if level sensor did not turn off | 200 | T37 | V4015 | V4035 |
| Receiver inside wash timer (12 sec) | 120 | T5 | V4016 | V4036 |
| Password for parameters change | 1234 |  | V4017 | V4037 |

Notes:

The values of timers are in 1/10 of a second. That means for 30 seconds the value should be set to 300.

The value of counters is set to No. + 1. That means for 5 times the value is set to 6.